# GridFlow Master Project Briefing

## Complete Knowledge

Document Purpose: Comprehensive briefing covering GridFlow's complete history, technical implementation, current state, strategic vision, and future roadmap. This document provides everything needed to understand the project from inception to future completion.

Last Updated: 2026-05-03
Project Phase: Phase C Complete, Strategic Scope Decision Pending
Repository: https://github.com/NoelyC123/Unitas-GridFlow
Local Path: /Users/noelcollins/Unitas-GridFlow

# PART 1: PROJECT ORIGIN AND VISION

## 1.1 Why GridFlow Exists

### The Problem GridFlow Solves

GridFlow was born from direct exposure to real-world friction in UK electrical distribution overhead line (OHL) survey-to-design workflows.

The problem is NOT:

- Bad field surveying

- Lack of GIS tools

- Poor CAD software

- Inadequate mobile apps

The problem IS: Survey data arrives at design teams in a weak, fragmented, unauditable form that wastes office time on clarification, interpretation, and evidence-hunting instead of actual engineering design work.

### Real-World Manifestation

What happens without GridFlow:

- Surveyor captures data in Field Maps, Trimble controller, or notebook

- Raw export arrives weeks later as CSV dump (coordinates, feature codes, sparse attributes)

- Designer opens file and discovers:

  - Heights missing or estimated

  - Material/condition not captured

  - Stay evidence absent at angle poles

  - EX/PR replacement relationships unclear

  - Span distances questionable

  - Clearance data missing

  - No provenance (who measured what, when, how accurate)

- Designer must:

  - Cross-reference field notes (if they exist)

  - Find marked-up plans (if they exist)

  - Chase photos (if they were taken)

  - Call surveyor (if they remember the job)

  - Make educated guesses

  - Flag for site revisit (costly, slow)

- Result: Design delayed, costs rise, office time wasted on survey archaeology

The key insight:

"The biggest inefficiency sits in the survey-to-design HANDOFF, not in the engineering design itself."

### What GridFlow Does About It

GridFlow acts as the intelligent gate between survey and design.

It:

- Ingests raw survey exports (multiple formats, coordinate systems)

- Parses electrical asset schema (poles, spans, stays, cables, context, equipment)

- Normalizes survey connectivity (EX↔PR replacement logic, route sequencing)

- Assesses completeness and design-readiness

- Flags missing/questionable evidence

- Visualizes asset lifecycle states

- Surfaces design blockers BEFORE design time is wasted

- Produces design-ready artefacts (sequenced routes, QA reports, review workspaces)

The value is trust and interpretation, not just parsing:

- "This pole exists, height measured 9.2m by surveyor J.Smith on 2024-03-15, GNSS accuracy ±0.15m"

- vs. "This pole exists, height unknown — field notes required before design"

- vs. "This proposed pole does not exist yet — specification is a design decision"

GridFlow tells designers exactly:

- What IS present in the digital file

- What is MISSING from the digital file

- What that PREVENTS (design blockers)

- Where to look for missing evidence (field notes, photos, plans)

## 1.2 Project Vision: What GridFlow Will Be When Fully Developed

### Vision Statement

GridFlow will be the professional electrical survey-to-design platform trusted by UK DNOs, contractors, and consultancies to transform raw OHL survey data into design-ready engineering intelligence.

### The Fully Realized Product (Long-term Vision)

When fully developed, GridFlow will:

#### 1. Capture Complete Survey Data (Stage 4+)

Field Capture Module:

- Tablet/mobile interface for structured data capture

- Offline-capable workflow (sync when connected)

- Photo capture with automatic asset linking

- Sketch/markup tools for complex situations

- Voice notes for rapid field observations

- GNSS integration with real-time accuracy display

- Multi-surveyor coordination (team workflows)

Asset Data Model (50+ fields per pole):

Physical Structure:

- Measured height (existing) / Specification (proposed)

- Pole class/strength rating

- Material (wood, concrete, steel, composite)

- Condition (good/fair/poor/unsafe)

- Decay/rot evidence

- Pole-top condition

- Lean direction/severity

- Birthmark/asset plate details

- Defect type (rot/split/burn/impact/woodpecker)

- Foundation/ground condition

- Action required (repair/replace/monitor)

Electrical/Network:

- Voltage carried (11kV, 33kV, 132kV)

- Circuit identifier

- Number of phases (1ph, 2ph, 3ph)

- Conductor type/size

- Pole-top arrangement

- Crossarm type

- Insulator type

- Equipment (transformer/switch/fuse/recloser/sectionalizer)

- Equipment ratings

- Asset plate information

- Earthing configuration

Mechanical Support:

- Stay required (yes/no, calculated from angle/loading)

- Stay existing (yes/no, field-verified)

- Stay type (angle/terminal/tee-off/tandem)

- Anchor location (GPS + bearing)

- Anchor type (screw/block/deadman/rock)

- Stay condition

- Stay insulator presence

- Strut/brace presence

Attachments/Third-Party:

- Telecoms attachment (BT/Virgin/other)

- Streetlight

- CCTV/traffic equipment

- Customer service connections

- Other utility crossings

Access/Environment/Safety:

- Access route description

- Gate/fence/wayleave constraints

- Landowner contact

- Vegetation/tree proximity

- Ground condition (stable/soft/flooded/steep)

- Traffic management requirements

- Protected species/environmental constraints

- Crossing clearances (road/rail/river/canal)

Evidence/Provenance:

- Full pole photo (linked, GPS-tagged)

- Pole-top photo

- Defect closeup photos

- Stay/anchor photos

- Asset plate photo

- Field sketches/markups

- Surveyor name

- Survey date/time

- GNSS accuracy (horizontal/vertical)

- Equipment used (controller model)

- Weather conditions

- Field notes/remarks

#### 2. Manage Asset Relationships (Stage 5)

Connectivity Model:

- Parent/child poles (tee-offs, spurs)

- Span connections (from-pole → to-pole)

- Stay anchor associations (pole ← stay → anchor)

- Cable/conductor runs (continuous circuits)

- Crossing context (pole proximity to roads/lines/rivers)

- Transformer/equipment serving areas

Route Intelligence:

- Auto-sequencing (start → end chain)

- Branch detection (main circuit + tee-offs)

- Loop identification (ring mains)

- Split-route handling (PoleCAD sections)

- Design chain optimization

Replacement Logic:

- EX↔PR pairing (proximity + designer confirmation)

- Retained pole identification

- Recovered/demolished pole tracking

- Repositioned pole detection (<5m offset)

- Upgrade-in-place recognition (same location, new spec)

#### 3. Provide Professional Engineering Workspace (Stage 4-5)

Map Viewer (Enhanced from Phase C):

- Multi-layer control (structural/electrical/mechanical/context/environment)

- Review filters (design blockers, missing evidence, QA flags)

- Symbol hierarchy (shape = asset type, color = condition/status)

- Lifecycle visualization (11 states with linking lines)

- Span analysis (distance, sag, clearance estimates)

- Context awareness (crossings, constraints, access)

- Evidence indicators (photo count, note presence, accuracy)

- Design action prompts (what to check, where to look)

Review Workspace:

- Issue prioritization (blocker/warning/info)

- Missing evidence tracking

- Designer decision recording (confirm/reject pairings, override flags)

- Annotation/markup tools

- Team collaboration (comments, assignments)

- Approval workflow (surveyor → reviewer → designer → approver)

Design Handoff:

- Sequenced route export (design-ready chain)

- PoleCAD-compatible format

- D2D (Design-to-Design) structured export

- GIS integration (upload to DNO spatial systems)

- PDF engineering pack (formal handover document)

- Issue register (what's missing, what's uncertain, what requires field revisit)

#### 4. Integrate with DNO Systems (Stage 6)

GIS Connectors:

- ArcGIS integration

- QGIS export

- DNO-specific spatial databases

- Field Maps round-trip (capture → GridFlow → validation → back to Field Maps)

Asset Management Systems:

- DNO asset register updates

- Work order integration

- Inspection/maintenance scheduling

- Historical record linkage

PoleCAD/Design Tools:

- Direct PoleCAD import (ready to design)

- Design feedback loop (as-built updates)

- Material optimization suggestions

- Loading/sag calculations pre-populated

#### 5. Apply DNO-Grade Compliance (Stage 6+)

Rulepack Expansion:

- Current: SPEN, SSEN, NIE, ENWL (basic 11kV)

- Future: All 14 UK DNOs

- Voltage-specific rules (11kV, 33kV, 132kV)

- Regional variations (Scottish hills, coastal exposure, urban congestion)

- Seasonal adjustments (ice/wind loading zones)

Engineering Standards:

- ENA G39 compliance (connection of microgeneration)

- ENA TS 43-8 (overhead line design)

- Clearance validation (EA TS 43-8, statutory minimums)

- Loading calculations (conductor sag, ice/wind)

- Stay design validation (angle, loading, anchor capacity)

Safety Gates:

- Unsafe structure detection (severe lean, rot, damage)

- Clearance violations (road/rail/building proximity)

- Voltage separation (HV crossing conflicts)

- Access constraints (traffic management requirements)

- Environmental flags (protected species, flood zones)

#### 6. Support Multi-User Deployment (Stage 7)

Authentication/Authorization:

- User accounts (surveyor/reviewer/designer/approver roles)

- Team/project-based access control

- DNO/contractor organization separation

- Audit logging (who changed what, when)

Collaboration:

- Multi-user projects

- Review/approval workflow

- Comment threads

- Assignment system

- Notification/alerting

Deployment Models:

- SaaS (hosted by Unitas/partner)

- On-premise (DNO-hosted)

- Hybrid (field capture SaaS, design review on-premise)

### The Fully Realized User Experience

Surveyor (Field):

- Opens GridFlow mobile app on tablet

- Selects project/circuit

- Walks route, capturing poles with structured form:

  - Tap pole location on map (GPS auto-captured)

  - Select type (existing/proposed/angle/terminal)

  - Measure height (laser rangefinder → auto-populate)

  - Photo: full pole (auto-tagged, GPS-linked)

  - Photo: pole-top (auto-tagged)

  - Select material (dropdown: wood/concrete/steel)

  - Rate condition (good/fair/poor/unsafe)

  - If angle: capture stay direction, photo anchor

  - Add remarks (voice note or text)

  - Submit → syncs when online

- Continues route, GridFlow auto-sequences

- At end of day: reviews completeness, flags uncertainties, submits for review

Reviewer (Office, Day 1):

- Opens GridFlow web interface

- Selects submitted survey

- Reviews map: auto-sequenced route, lifecycle visualization, QA flags

- Clicks pole with warning: "Height estimated, not measured"

  - Checks photo: can't clearly see pole-top

  - Adds comment: "Surveyor: please revisit, measure height with rangefinder"

  - Assigns back to surveyor

- Reviews angle pole: "Stay evidence not captured"

  - Checks field notes: no mention of stay

  - Checks photos: no anchor visible

  - Flags: "Site revisit required - stay configuration unknown"

- Reviews EX↔PR pairings: 15 suggested matches

  - Confirms 12 (proximity + logic clear)

  - Rejects 2 (wrong poles paired)

  - Leaves 1 uncertain (designer decision)

- Approves survey for design (with flagged items)

Designer (Office, Day 2-5):

- Opens GridFlow design workspace

- Imports survey (sequenced route auto-loaded)

- Reviews issue register:

  - 3 poles require site revisit (heights/stays missing)

  - 5 poles need specification (proposed poles)

  - 2 span clearances need verification (road crossings)

- Filters map: "Design blockers only"

  - Sees 3 critical issues

  - Adds to site visit request

- Begins design on ready sections:

  - Pole specifications (uses measured heights, conditions, loadings)

  - Conductor selection (uses voltage, span distances, clearance requirements)

  - Stay design (uses captured angles, existing stay evidence)

  - Material optimization (uses condition ratings, replacement priorities)

- Exports to PoleCAD: ready to design (no survey archaeology needed)

- Completes design in 60% less time (no clarification delays)

DNO Approver (Office, Day 6):

- Reviews completed design

- Checks GridFlow evidence pack:

  - Survey completeness: 95% (5% flagged for revisit)

  - Design-readiness: READY (all blockers resolved)

  - QA status: 2 warnings (acceptable), 0 failures

  - Evidence quality: HIGH (measured data, photos, GNSS <0.2m)

- Approves design for construction

- GridFlow updates asset register (as-designed records)

Construction Team (Site, Weeks 1-4):

- Accesses GridFlow mobile (construction mode)

- Views design route on map

- At each pole:

  - Sees design specification

  - Compares to as-found (GridFlow survey data visible)

  - Records as-built (actual installed spec)

  - Photos installed pole

- Submits as-built records

- GridFlow updates asset register (final as-built)

### The Business Vision

GridFlow will generate revenue through:

1. SaaS Subscriptions:

- Surveyor tier: £50-100/user/month (field capture, basic QA)

- Designer tier: £150-250/user/month (full workspace, design handoff)

- Enterprise tier: £500-1000/user/month (DNO deployment, API access, custom rulepacks)

2. Professional Services:

- Survey data cleanup/migration (£2,000-10,000/project)

- Custom rulepack development (£5,000-20,000/DNO)

- Training/onboarding (£1,000-5,000/team)

- Consultancy (survey process optimization, workflow design)

3. Integration/Deployment:

- On-premise deployment (£50,000-200,000 one-time + £10,000-50,000/year support)

- GIS/asset system integration (£20,000-100,000/connector)

- Custom feature development (£50,000-500,000 depending on scope)

Target Market:

- 14 UK DNOs (electricity distribution network operators)

- 50+ major electrical contractors

- 200+ electrical engineering consultancies

- International expansion: Ireland, Australia, New Zealand, North America

Market Size (UK):

- 14 DNOs × £100,000/year avg = £1.4M/year (conservative)

- 50 contractors × £20,000/year avg = £1M/year

- 200 consultancies × £5,000/year avg = £1M/year

- Total addressable: £3.4M/year in UK alone

5-Year Vision:

- Year 1: Prove value with 2-3 DNOs, 5-10 contractors (£200K revenue)

- Year 2: Scale to 6-8 DNOs, 20 contractors (£800K revenue)

- Year 3: 10+ DNOs, 40+ contractors, international pilot (£2M revenue)

- Year 4: Market leader position, international expansion (£5M revenue)

- Year 5: Platform maturity, API ecosystem, acquisition target (£10M+ revenue)

## 1.3 Current Reality vs. Vision

Where GridFlow is NOW (May 2026):

Product Status: Phase C complete — professional QA + basic lifecycle intelligence

- 12-field popup data model (not 50-field)

- Map intelligence (filters, lifecycle viz, span analysis, stay detection)

- Design-readiness assessment (not field capture)

- PDF QA reports (not design handoff packs)

- Internal/consultancy tool (not commercial SaaS)

What's Built: ✅ Multi-format intake (Trimble, controller, structured CSV) ✅ Multi-coordinate system (OSGB, Irish Grid, WGS84) ✅ Electrical schema parsing (6 asset families) ✅ Survey connectivity (EX↔PR, sequencing) ✅ QA validation (10 check types, 4 DNO rulepacks) ✅ Map viewer (15 filters, lifecycle viz, span analysis) ✅ Design-readiness reports ✅ 300+ passing tests ✅ Real operational validation (P011, Gordon, Bellsprings)

What's NOT Built Yet: ❌ Field capture (mobile/tablet interface) ❌ 50-field data model (only 12 fields currently) ❌ Photo upload/management ❌ Offline workflows ❌ Asset relationship model (parent/child, connectivity) ❌ Multi-user collaboration ❌ GIS integration ❌ PoleCAD export ❌ Commercial deployment infrastructure ❌ Authentication/authorization ❌ API for third-party integration

The Gap:

- Vision: Professional electrical survey platform (6-12 months development)

- Current: Enhanced pre-CAD QA gatekeeper (operational, proven)

- Decision Point: Phase C2/D (2-4 weeks, modest expansion) vs. Stage 4 (6-12 months, full vision)

# PART 2: TECHNICAL IMPLEMENTATION HISTORY

## 2.1 Technology Stack

### Core Runtime

Backend:

- Python 3.13 (current, tested on 3.11+)

- Flask (web framework, app factory pattern)

- pandas (tabular data processing)

- geopandas (spatial data handling)

- shapely (geometry operations)

- pyproj (coordinate system transformations)

- ReportLab (PDF generation)

Frontend:

- Leaflet.js (map rendering, marker controls)

- Bootstrap 5 (UI framework, responsive layout)

- Vanilla JavaScript (no heavy frameworks, ES6+)

Quality/Testing:

- pytest (test framework, 300+ tests)

- pytest-cov (coverage reporting)

- Ruff (linting, formatting)

- pre-commit (hooks for quality gates)

- GitHub Actions (CI/CD pipeline)

Development Environment:

- macOS (primary development machine)

- VS Code / Cursor (code editing)

- Git (version control)

- Virtual environment (.venv312)

### Architecture Overview

Flask App Structure:

app/

├── __init__.py              # App factory, blueprint registration

├── routes/

│   ├── api_upload.py        # File upload endpoint

│   ├── api_intake.py        # CSV parsing, QA processing

│   ├── map_preview.py       # Map viewer routes, runtime enrichment

│   ├── pdf_reports.py       # PDF generation

│   ├── pages.py             # Static pages (home, about, help)

│   ├── jobs.py              # Job browser, job detail

│   ├── projects.py          # Project management

│   ├── review.py            # Review workspace

│   ├── field_capture.py     # Stage 4 proof-of-concept (not production)

│   └── d2d_export.py        # Design-to-Design export

├── services/

│   ├── qa_engine.py         # QA validation engine (~1,095 lines)

│   ├── controller_intake.py # Controller dump parsing (~816 lines)

│   ├── route_sequencer.py   # Route auto-sequencing (~691 lines)

│   ├── span_generator.py    # Span distance/anomaly detection (~542 lines)

│   ├── electrical_schema.py # Asset taxonomy (~420 lines)

│   ├── survey_connectivity.py # EX↔PR matching (~380 lines)

│   ├── cable_generator.py   # Cable/UG asset handling

│   ├── asset_classifier.py  # Asset type classification

│   ├── context_crossing.py  # Crossing enrichment

│   └── field_ownership.py   # Field presence/completeness tracking

├── static/

│   ├── js/

│   │   └── map-viewer.js    # Map intelligence (~2,809 lines) ⚠️ SENSITIVE

│   └── css/

│       └── map-viewer.css

├── templates/

│   ├── map_viewer.html      # Map viewer UI

│   ├── review_page.html     # Review workspace UI

│   ├── job_detail.html

│   └── ...

└── dno_rules.py             # DNO-specific rulepacks

Data Flow:

1. Upload CSV → api_upload.py (save file)

2. Import/Process → api_intake.py:

- controller_intake.py (parse CSV, normalize columns)

- electrical_schema.py (classify assets)

- survey_connectivity.py (EX↔PR matching)

- route_sequencer.py (auto-sequence route)

- span_generator.py (span distance, anomaly detection)

- qa_engine.py (run validation checks)

- Generate outputs:

* meta.json (summary, completeness, design-readiness)

* issues.csv (QA flags, warnings, blockers)

* map_data.json (point features, basic attributes)

* sequenced_route.json (design chain)

3. View Map → map_preview.py:

- Load map_data.json, sequenced_route.json

- Runtime enrichment (lifecycle states, popup data, span features)

- Render map_viewer.html

- map-viewer.js handles UI (filters, popups, lifecycle viz)

4. Download PDF → pdf_reports.py (ReportLab generation)

5. Browse Jobs → jobs.py (list all processed jobs)

### Coordinate System Handling

Supported Systems:

- OSGB36 (British National Grid, EPSG:27700)

  - England, Scotland, Wales

  - Easting/Northing in meters

  - Origin: Ordnance Survey datum

- Irish Grid TM65 (EPSG:29902)

  - Northern Ireland, Republic of Ireland

  - Easting/Northing in meters

  - Origin: Geodetic Survey of Ireland

- WGS84 (GPS, EPSG:4326)

  - Global latitude/longitude

  - Converted on intake to regional grid

Conversion Pipeline:

python

# pyproj handles all transformations

from pyproj import Transformer

# Example: Irish Grid → WGS84 for Leaflet display

transformer = Transformer.from_crs("EPSG:29902", "EPSG:4326", always_xy=True)

lat, lng = transformer.transform(easting, northing)

Detection Logic:

- Easting 0-700000, Northing 0-1300000 → OSGB

- Easting 0-500000, Northing 0-500000 → Irish Grid

- Lat/Lng → WGS84

### Data Model (Current - Phase C)

Job Outputs (per processed job):

1. meta.json (summary)

json

{

"job_id": "J12946",

"filename": "2814_474c_raw_trimble_export.csv",

"processed_at": "2026-04-15T14:32:01",

"total_records": 91,

"mapped_features": 91,

"coordinate_system": "OSGB36",

"design_readiness": "PARTIALLY READY",

"readiness_reasons": [

"Height coverage: 19/91 poles (21%)",

"Material data: 0/91 poles (0%)",

"Stay/clearance data incomplete"

],

"circuit_summary": {

"total_poles": 67,

"existing_poles": 45,

"proposed_poles": 22,

"angle_poles": 12,

"stays": 8,

"context_features": 24

},

"completeness": {

"height": "21%",

"material": "0%",

"location": "12%",

"condition": "0%"

},

"sequence_summary": {

"sequenced_items": 64,

"confidence_high": 32,

"confidence_medium": 32,

"confidence_low": 0,

"detached_items": 3

}

}

2. issues.csv (QA flags)

csv

issue_id,point_id,severity,code,category,message,recommendation

1,47,WARNING,EXPOLE_HEIGHT_EST,completeness,"Existing pole height estimated (not measured)","Verify height from field notes or plan evidence"

2,52,BLOCKER,ANGLE_NO_STAY,mechanical,"Angle pole with no stay evidence captured","Check field notes/photos for stay configuration"

3,159-160,INFO,SPAN_VERY_SHORT,sequencing,"Very short span (4.6m) - possible duplicate or repositioned pole","Review coordinates and field notes"

...

3. map_data.json (point features)

json

{

"type": "FeatureCollection",

"features": [

{

"type": "Feature",

"geometry": {

"type": "Point",

"coordinates": [-122.4194, 37.7749]  // lng, lat (WGS84)

},

"properties": {

"point_id": "47",

"record_type": "EXpole",

"function": "Angle",

"height": 9.2,

"material": null,

"condition": null,

"easting": 365377.835,

"northing": 643824.697,

"elevation": 127.3,

"coordinate_system": "OSGB36",

"remarks": "ex pole",

"qa_status": "WARN",

"lifecycle_state": "existing",  // Phase C addition

"has_stay_evidence": false,     // Phase C addition

"is_angle_pole": true            // Phase C addition

}

},

// ... more features

]

}

4. sequenced_route.json (design chain)

json

{

"sequence_id": "SEQ_J12946",

"total_items": 64,

"chain": [

{

"position": 1,

"point_id": "1",

"record_type": "EXpole",

"confidence": "HIGH",

"distance_to_next": 45.2,

"bearing_to_next": 87.3

},

{

"position": 2,

"point_id": "2",

"record_type": "EXpole",

"confidence": "HIGH",

"distance_to_next": 52.8,

"bearing_to_next": 89.1

},

// ...

],

"detached_items": [

{"point_id": "999", "reason": "Isolated context feature"}

],

"matches": [

{

"ex_pole": "47",

"pr_pole": "48",

"distance": 15.2,

"status": "suggested",

"reviewed": false

}

]

}

## 2.2 Phase Development History

### Phase 0: Pre-Project (2023-2024)

Real-world observations:

- Working on UK OHL refurbishment projects

- Seeing survey-to-design handoff friction repeatedly

- Designers spending 40-60% of time on "survey archaeology"

- Missing heights, stays, materials forcing site revisits

- Weak controller exports requiring manual interpretation

- No structured gate between survey and design

Key insight: The problem is not individual tools (Field Maps works, PoleCAD works), but the handoff gap between them.

### Phase 1: MVP Build (Early 2025)

Goal: Prove basic intake → QA → output flow works

Delivered:

- Flask app structure (blueprint-based routes)

- CSV upload endpoint

- Basic controller parsing (Trimble format)

- Placeholder QA rules (coordinate bounds, duplicate IDs)

- Map viewer (Leaflet, basic point display)

- PDF report generation (ReportLab)

- Job storage (filesystem-based)

Test Count: ~15-20 tests

Validation: Synthetic test files only

Key Learning: Basic flow works, but rules are too generic, intake is brittle, no real validation

### Phase 2: Domain Intelligence (Spring 2025)

Goal: Move from placeholder rules to domain-aware validation

Phase 2A: Input Schema Normalization

Problem: Real Trimble/controller exports have:

- Capitalized headers ("Easting" vs. "easting")

- Spaced headers ("Point Number" vs. "point_number")

- Abbreviated names ("Ht" vs. "height")

- OSGB-specific aliases ("Northing" vs. "Y")

Solution:

- Added column name normalization in api_intake.py:

  - Strip whitespace

  - Lowercase

  - Replace spaces with underscores

  - Apply alias mapping (e.g., "ht" → "height", "northing" → "y")

- Expanded alias coverage for all core fields

Test Count: 35 → 38 tests

Validation: Real Gordon/Bellsprings files (first time)

Key Learning: Real files are messier than expected, normalization essential

Phase 2B: Electrical Schema + Connectivity (Spring 2025)

Problem: Generic "pole" classification misses engineering context

Solution Created:

- electrical_schema.py (~420 lines)

  - 6 asset families: poles, spans, stays, cables, context, equipment

  - Equipment taxonomy: transformers, switches, fuses, reclosers, sectionalizers

  - Connectivity types: overhead_line, underground_cable, aerial_bundled_conductor

- survey_connectivity.py (~380 lines)

  - EX↔PR replacement logic (proximity-based matching)

  - Span sequencing (route chain detection)

  - Stay/anchor associations

Test Count: 38 → 150+ tests

Validation: 2814_474, 2814_513, Gordon families

Key Learning: Domain taxonomy is critical — feature codes alone insufficient

Phase 2C: Route Sequencing (Spring 2025)

Problem: Survey points are unordered — designers need design chains

Solution:

- route_sequencer.py (~691 lines)

  - Auto-sequencing algorithm (nearest-neighbor with bearing constraints)

  - Confidence levels (high/medium/low)

  - Branch detection (tee-offs)

  - Detached item identification

Output: sequenced_route.json

Test Count: 150+ → 200+ tests

Validation: Gordon split-file testing (PR1 + PR2)

Key Learning: Auto-sequencing works well for simple routes, struggles with complex tee-offs

Phase 2D: QA Engine Enhancement (Spring 2025)

Problem: Placeholder rules don't catch real design blockers

Solution:

- qa_engine.py expanded (~1,095 lines)

  - 10 check types:

    - Coordinate consistency (bounds, plausibility)

    - Duplicate coordinate pairs (GPS error detection)

    - Height validation (range, missing, estimated)

    - Span distance logic (short/long/missing poles)

    - Angle pole stay evidence

    - Material/condition completeness

    - EX/PR replacement clarity

    - Clearance proximity (context features)

    - Feature code validity

    - Sequence confidence

  - 4 DNO rulepacks:

    - SPEN_11kV (Scottish Power)

    - SSEN_11kV (Scottish & Southern)

    - NIE_11kV (Northern Ireland Electricity)

    - ENWL_11kV (Electricity North West)

Test Count: 200+ → 280+ tests

Validation: Real jobs (J12946, P005/F001, Gordon, Bellsprings)

Key Learning: Real issues (REPL_PAIR, ANGLE_NO_STAY, SPAN_VERY_SHORT) emerge consistently across jobs

### Phase 3: Operational Validation (April 2026)

Goal: Test on real designer use, collect operational feedback

P011 Operational Feedback:

- Real 11kV refurbishment job

- Designer (real user, not test) used GridFlow

- 6 operational blockers identified:

  - Context features invisible (filter defaulted off)

  - "Height: not captured" misleading for proposed poles

  - Symbols not distinct at map zoom

  - Stay evidence missing at angle poles

  - Span distance unclear

  - Popups insufficient for design decisions

Key Learning: Tool works, but UX/trust issues prevent designer confidence

### Phase C: Map Intelligence (April-May 2026)

Goal: Resolve 6 operational blockers, make tool design-ready

Phase C1: Feature-Type Filtering + Blank Field Framework

Delivered:

- 15 filter buttons (existing, proposed, angle, stays, context, missing heights, etc.)

- Conditional blank field display:

  - Existing pole: "Height: not captured (review required)" ⚠️

  - Proposed pole: "Height: not yet specified (design decision)" ℹ️

  - Context record: Height field hidden entirely

- Symbol hierarchy (shape = asset type, color = QA status)

- Popup section organization (8 sections: Identity, Physical, Electrical, Mechanical, Location, Context, QA/Review, Lifecycle)

Test Count: 280+ → 290+ tests

Validation: P011 visual acceptance testing

Blocker Resolved: #1 (context invisible), #2 (misleading height wording), #6 (insufficient popups)

Phase C2: Asset Lifecycle Visualization

Delivered:

- 11 lifecycle states:

  - existing, proposed, retained, recovered, replacement, repositioned

  - unmatched_ex, unmatched_pr, suggested, confirmed, context

- EX↔PR linking lines:

  - Dashed = suggested match (proximity-based, not reviewed)

  - Solid = confirmed match (designer-verified)

  - Distance label on line

- Lifecycle toggle layer (on/off control)

- Lifecycle section in popups

Test Count: 290+ → 295+ tests

Validation: P011, Gordon EX/PR pairing scenarios

Blocker Resolved: #3 (symbol clarity — lifecycle states now visually distinct)

Phase C3: Stay Evidence at Angle Poles

Delivered:

- Angle pole detection:

  - Function="Angle" OR bearing deviation >10°

- Stay record scanning (20m radius)

- Stay type classification (angle/terminal/tee-off/tandem)

- Missing stay warnings:

  - "⚠️ Angle pole — stay evidence not captured. Check field notes, photos or plan evidence."

- Found stay display:

  - "✓ Stay Evidence: Angle Stay @ 12.4m, bearing 215°"

- Filter: "Angle Poles Missing Stays" (design blocker focus)

Test Count: 295+ → 298+ tests

Validation: Gordon, Bellsprings (real angle poles)

Blocker Resolved: #4 (stay evidence missing — now detected and flagged)

Phase C4: Span Anomaly Detection + Crossing Context

Delivered:

- 3D span distance calculation (includes elevation)

- Span anomaly classification:

  - <10m: Probable duplicate (GPS error)

  - 100-300m (11kV): Extended span (verify sag)

  - 500m (11kV): Critical gap (probable missing pole)

- Span lines + distance labels on map

- Crossing context enrichment:

  - Road Crossing: "Critical clearance check required" (HIGH priority)

  - 11kV Crossing: "Voltage separation required" (HIGH)

  - 33kV Crossing: "Critical voltage clearance" (CRITICAL)

  - 110kV Crossing: "Specialist review required" (CRITICAL)

- Filters: "Show Span Anomalies", "Show Crossings Requiring Clearance"

Test Count: 298+ → 300+ tests

Validation: P011 (span distance visible), Gordon (crossing priorities)

Blocker Resolved: #5 (span unclear — now visible with distance labels + anomaly flags)

Phase C Summary:

- Duration: April-May 2026

- Test Growth: 280 → 300+ tests (+7%)

- Blockers Resolved: 6/6 from P011 operational feedback

- New Capabilities: 15 filters, 11 lifecycle states, stay detection, span analysis

- Status: COMPLETE ✅

### Phase D/Stage 4: NOT STARTED (Future Work)

What comes next depends on strategic decision:

If Phase C2/D (Professional QA + Display):

- Map UX refinement (marker size, symbol polish, layer panel split)

- Add 10-15 key survey fields (12 → 25 field popup)

- Asset-specific popup layouts

- Timeline: 2-4 weeks

If Stage 4 (Full Survey Platform):

- Complete 50-field data model

- Evidence management (photo upload/linking)

- Mobile/tablet capture interface

- Asset relationship model

- Timeline: 6-12 months

- Requires customer validation FIRST

## 2.3 Key Technical Decisions and Trade-offs

### Decision 1: Filesystem Storage vs. Database

Chosen: Filesystem-based job storage

Why:

- Simple deployment (no DB setup)

- Easy backup/migration (copy folder)

- Direct file access (no ORM complexity)

- Human-readable outputs (JSON, CSV)

- Sufficient for current scale (100s of jobs, not 1000s)

Trade-off:

- Harder to query/search across all jobs

- No ACID transactions

- Scaling ceiling at ~10,000 jobs

Future: Will need database for multi-user Stage 6+ (PostgreSQL/PostGIS likely)

### Decision 2: Runtime Enrichment vs. Full Persistence

Chosen: Hybrid — basic attributes in map_data.json, enrichment in map route

Why:

- Lighter stored JSON (smaller files, faster upload)

- Flexibility (can change enrichment logic without re-processing)

- Separation of concerns (intake vs. display)

Trade-off:

- Map route is "source of truth" for lifecycle/popup data (not stored JSON)

- Downstream tools can't rely on stored JSON alone

- Requires understanding of enrichment pipeline

Future: Stage 4+ will need full persistence (API access, offline use)

### Decision 3: Coordinate System Auto-detection vs. User Selection

Chosen: Auto-detection based on coordinate ranges

Why:

- Better UX (no manual selection)

- Works for 95% of files (OSGB, Irish Grid ranges don't overlap)

- Fallback to WGS84 if ambiguous

Trade-off:

- Can misdetect if coordinates are edge-of-range

- No override mechanism (yet)

Future: Stage 4 will add manual override + validation warning

### Decision 4: Monolithic map-viewer.js vs. Modular Components

Chosen: Monolithic (~2,809 lines)

Why:

- Faster initial development (no build tooling)

- Vanilla JS (no framework lock-in)

- All logic in one place (easier to understand)

Trade-off:

- Large file (hard to navigate)

- Risky for AI-driven edits (fragile)

- No component reuse

Future: Stage 5 will need refactor (Vue/React components, build tooling)

### Decision 5: Pytest vs. Other Test Frameworks

Chosen: pytest

Why:

- Python standard (widely used)

- Simple syntax (no boilerplate)

- Good fixtures/parametrization

- Coverage integration

Trade-off:

- No browser automation (Playwright/Selenium separate)

- Backend-focused (frontend tests manual)

Future: Stage 5 will add Playwright for UI testing

# PART 3: DOMAIN KNOWLEDGE

## 3.1 UK Electrical Distribution Context

### DNO Landscape

14 UK DNOs (Distribution Network Operators):

Scotland:

- SPEN (Scottish Power Energy Networks)

- SSEN (Scottish & Southern Energy Networks — North)

England:

- SSEN (South)

- UKPN (UK Power Networks) — London, South East, East

- NPG (Northern Powergrid) — North East, Yorkshire

- ENWL (Electricity North West)

- WPD/National Grid (Western Power Distribution) — Midlands, South West, Wales

Northern Ireland:

- NIE Networks (Northern Ireland Electricity)

Voltage Levels:

- 11kV (distribution, most common OHL)

- 33kV (primary distribution, higher capacity)

- 132kV (transmission/distribution boundary)

- 400kV (transmission, National Grid, not DNO)

GridFlow Focus: 11kV distribution OHL (most surveyed, most design work)

### Asset Types (Electrical Schema)

1. Poles/Support Structures

Types:

- Existing (EX) — currently in field, may be retained/replaced/recovered

- Proposed (PR/Pol) — new pole to be installed

- Angle — route deviation >10°, requires stays

- Terminal — end of line, requires terminal stay

- Intermediate — straight-through, no angle

- Tee-off — branch point, requires tee-off stay

Materials:

- Wood (most common, older networks)

- Concrete (modern standard)

- Steel (high-load, special applications)

- Composite (modern, lightweight)

Condition:

- Good (no defects, full service life remaining)

- Fair (minor defects, serviceable, monitor)

- Poor (significant defects, near end-of-life, plan replacement)

- Unsafe (structural failure risk, immediate action required)

Heights:

- 7-12m typical for 11kV distribution

- Measured vs. estimated (measured = better)

- Missing height = design blocker (can't calculate clearances)

2. Spans (Overhead Line Connections)

Typical Lengths:

- 11kV: 40-100m normal, 100-300m extended, >300m rare (probable missing pole)

- 33kV: 60-200m normal, >500m rare

- 132kV: 100-400m normal

Anomalies:

- <10m: Probable GPS error or duplicate capture

- 500m (11kV): Probable missing intermediate pole

- Inconsistent bearing: Route deviation, check for angle pole

Design Factors:

- Conductor sag (longer span = more sag)

- Clearance (ground, road, buildings, trees)

- Loading (ice, wind, tension)

3. Stays/Anchors (Mechanical Support)

When Required:

- Angle poles (>10° deviation)

- Terminal poles (end of line)

- Tee-offs (branch points)

- High-load poles (transformers, heavy conductors)

Types:

- Angle stay — opposes line tension change at angle

- Terminal stay — opposes line tension at end

- Tee-off stay — opposes branch tension

- Tandem stay — multiple stays in line for extra support

Anchor Types:

- Screw anchor (most common, augers into ground)

- Block anchor (concrete block buried)

- Deadman anchor (timber beam buried)

- Rock anchor (drilled into rock)

Stay Insulators:

- Required to break electrical path to ground

- Missing insulator = electrical hazard

Missing Stay Evidence = Design Blocker:

- Can't design pole without knowing stay configuration

- Can't calculate loading without stay support

- Site revisit required if no field evidence (notes/photos)

4. Cables (Underground/Aerial)

Types:

- Underground cable (UG, buried)

- Aerial bundled conductor (ABC, overhead, insulated)

- Service drops (pole to customer)

Relevance to GridFlow:

- Often captured as context features

- Less design-critical than poles/spans

- Future Stage 4+ feature (cable-pole transitions)

5. Context Features (Non-Electrical Assets)

Types:

- Roads (crossing clearance critical)

- Railways (DNO/Network Rail coordination required)

- Rivers/canals (crossing permits)

- Fences/gates (access constraints)

- Buildings (clearance, wayleaves)

- Trees (vegetation management)

- Other utilities (BT, gas, water, telecoms)

Crossing Priorities:

- CRITICAL: 110kV/132kV power lines, railways, motorways

- HIGH: Roads, 33kV lines, rivers

- MEDIUM: BT/telecoms, 11kV lines, minor roads

- LOW: Fences, gates, field boundaries

Why Context Matters:

- Clearance requirements (statutory minimums)

- Access planning (how to reach poles)

- Wayleave permissions (landowner consent)

- Construction constraints (traffic management, permits)

6. Equipment (Transformers, Switchgear)

Types:

- Transformer (voltage conversion, 11kV → 400V)

- Switch (circuit isolation, reconfiguration)

- Fuse (overcurrent protection)

- Recloser (automatic fault clearing)

- Sectionalizer (fault location)

Relevance to GridFlow:

- Equipment presence increases pole loading

- Requires larger pole, additional stays

- Access requirements (maintenance)

- Currently captured sparsely (future enhancement)

## 3.2 Survey Workflows (Real-World)

### Traditional Survey Workflow (Without GridFlow)

Week 1: Field Survey

- Surveyor walks route with GNSS controller (Trimble/Leica)

- Captures pole positions (GPS, feature code)

- Estimates heights visually (no laser rangefinder)

- Records minimal attributes (often just coordinates + code)

- Takes some photos (maybe, if remembered)

- Makes field notes in notebook (variable quality)

- Marks up plan with red pen (poles, angles, issues)

Week 2: Office Processing

- Surveyor downloads controller to CSV

- Emails CSV to design team (no QA, no review)

- CSV format varies (Trimble vs. Leica vs. manual)

- Headers inconsistent (capitalized, spaced, abbreviated)

- No completeness check

- No design-readiness assessment

Week 3: Design Starts

- Designer opens CSV in Excel

- Discovers:

  - Heights missing or estimated (80% of poles)

  - Material/condition not captured

  - Stay evidence absent at angle poles

  - Duplicate coordinates (GPS errors)

  - Unclear EX/PR relationships

  - Span distances questionable

- Designer emails surveyor: "Can you check heights for poles 15-24?"

- Surveyor: "That job was 3 weeks ago, I don't remember, I'll check field notes"

- Field notes: "Pole 15 approx 10m, pole 16 similar, pole 17 broken top"

- Designer: "I need measured heights, not estimates"

- Surveyor: "I'll have to revisit site"

Week 4: Site Revisit (£500-2000 cost)

- Surveyor returns to site

- Measures heights properly (laser rangefinder)

- Checks stay configurations

- Takes proper photos

- Returns to office, updates CSV manually

Week 5: Design Continues

- Designer imports updated CSV

- Still missing material/condition for many poles

- Proceeds with design, makes assumptions

- Flags uncertainties for construction team

Week 6-8: Design Completion

- Design finalized with caveats

- As-found vs. as-designed discrepancies noted

- Construction team will verify on site

Total Timeline: 6-8 weeks, survey-to-design (with revisit) Total Cost: Survey £2K + revisit £1K + design delays £3K = £6K+ project overhead

### Improved Workflow (With GridFlow)

Week 1: Field Survey (Same)

- Surveyor captures data (as before)

- Returns to office, downloads CSV

Week 1 (Day 5): GridFlow Processing (30 minutes)

- Surveyor uploads CSV to GridFlow

- GridFlow processes:

  - Parses format (Trimble/Leica/manual)

  - Normalizes columns

  - Classifies assets

  - Sequences route

  - Runs QA checks

  - Generates completeness report

- Surveyor reviews map:

  - Sees 45 poles, 12 angle, 8 context features

  - Filters: "Missing Heights" → 36 poles flagged

  - Filters: "Angle Poles Missing Stays" → 9 poles flagged

  - Reviews issues: 18 warnings, 3 blockers

- Surveyor decision:

  - Minor: proceed to design (document gaps)

  - Major: site revisit this week (while memory fresh)

Week 1 (Day 6): Targeted Revisit (If Needed)

- Surveyor returns to site with focused list:

  - 9 angle poles (measure heights, photograph stays)

  - 3 unclear EX/PR pairs (verify coordinates)

  - 2 road crossings (measure clearances)

- Captures missing data

- Updates survey

- Re-processes in GridFlow

- Issues resolved: 18 → 3 warnings, 0 blockers

Week 2: Design Starts (Immediately)

- Designer receives GridFlow package:

  - Sequenced route (design chain ready)

  - Completeness report (knows what's missing)

  - Issue register (3 warnings documented)

  - Map workspace (visual review)

- Designer reviews map:

  - Heights: 80% measured, 20% documented as estimated

  - Angles: all have stay evidence or flagged for review

  - EX/PR pairs: 12 suggested, 10 confirmed, 2 require field decision

  - Spans: all reasonable, no duplicates

- Designer proceeds with confidence:

  - Uses measured data

  - Documents assumptions for estimated data

  - Flags 3 warnings for construction verification

  - No delays, no uncertainty paralysis

Week 3-4: Design Completion

- Design completed 40% faster (no survey archaeology)

- Higher confidence (measured data, documented gaps)

- Fewer as-found discrepancies (better survey quality)

Total Timeline: 3-4 weeks, survey-to-design (no revisit) Total Cost: Survey £2K + GridFlow £200 = £2.2K Savings: £3.8K per job (60% reduction)

## 3.3 Feature Codes (Trimble/Controller Conventions)

Common Feature Codes:

Poles:

- EXpole — existing pole

- Pol / PR / PRpole — proposed pole

- Angle — angle pole

- Term / Terminal — terminal pole

- Tee — tee-off pole

Stays/Anchors:

- Stay — stay (generic)

- Anchor / Anc — anchor

- StayAngle — angle stay

- StayTerm — terminal stay

Context:

- Road — road crossing

- Gate — gate/fence

- BT / Telecom — telecoms crossing

- 11xing — 11kV line crossing

- 33xing — 33kV line crossing

- 110xing / HVxing — high voltage crossing

- Tree — tree/vegetation

- Wall / Fence — boundary

Equipment:

- TX / Transformer — transformer

- Switch / SW — switch

- Fuse — fuse

Cables:

- UG / Cable — underground cable

- ABC — aerial bundled conductor

Special:

- Base / PRS — base station (survey control, NOT pole)

- CP / CheckPoint — check point (survey QA)

Note: Feature codes are hints, not final truth — GridFlow classifies based on code + context

# PART 4: CURRENT STATE DEEP DIVE

## 4.1 What Works Right Now (Phase C Complete)

### Core Processing Pipeline

1. Multi-Format CSV Intake

Supported Formats:

- Trimble GNSS controller export

- Leica controller export

- Structured CSV (PoleCAD-style)

- Raw controller dump (job header + base station + variable-length records)

Column Normalization:

python

# Example: controller_intake.py

def _normalize_dataframe(df):

"""

Normalize column names:

- Strip whitespace

- Lowercase

- Replace spaces with underscores

- Apply alias mapping

"""

df.columns = df.columns.str.strip().str.lower().str.replace(' ', '_')

alias_map = {

'point_number': 'point_id',

'pt_no': 'point_id',

'ht': 'height',

'northing': 'y',

'easting': 'x',

'feature': 'feature_code',

# ... 50+ aliases

}

df.rename(columns=alias_map, inplace=True)

return df

Coordinate System Detection:

python

def detect_coordinate_system(df):

"""

Auto-detect based on coordinate ranges:

- OSGB: E: 0-700K, N: 0-1.3M

- Irish Grid: E: 0-500K, N: 0-500K

- WGS84: Lat: -90 to 90, Lng: -180 to 180

"""

if 'easting' in df.columns:

e_min, e_max = df['easting'].min(), df['easting'].max()

n_min, n_max = df['northing'].min(), df['northing'].max()

if e_max < 500000 and n_max < 500000:

return 'Irish_Grid_TM65'

elif e_max < 700000 and n_max < 1300000:

return 'OSGB36'

return 'WGS84'

2. Electrical Schema Classification

Asset Families (6):

python

# electrical_schema.py

ASSET_FAMILIES = {

'pole_support': ['EXpole', 'Pol', 'PR', 'PRpole', 'Angle', 'Terminal', 'Tee'],

'span_connection': ['Span', 'Conductor', 'OHL'],

'stay_anchor': ['Stay', 'Anchor', 'StayAngle', 'StayTerm'],

'cable_ug': ['UG', 'Cable', 'ABC'],

'context_feature': ['Road', 'Gate', 'BT', '11xing', '33xing', 'Tree', 'Fence'],

'equipment': ['TX', 'Transformer', 'Switch', 'Fuse', 'Recloser']

}

def classify_record(row):

"""

Classify based on feature_code + function + remarks.

Returns primary_type, secondary_type, confidence.

"""

code = row.get('feature_code', '').lower()

function = row.get('function', '').lower()

remarks = row.get('remarks', '').lower()

# Check each family

for family, codes in ASSET_FAMILIES.items():

if any(c.lower() in code or c.lower() in function for c in codes):

return {

'primary_type': family,

'record_type': code,

'confidence': 'HIGH'

}

# Fallback to context

return {

'primary_type': 'context_feature',

'record_type': 'unknown',

'confidence': 'LOW'

}

3. Survey Connectivity Normalization

EX↔PR Replacement Matching:

python

# survey_connectivity.py

def detect_ex_pr_pairs(records, max_distance=50):

"""

Find probable EX↔PR replacement pairs.

Returns list of matches with distance, status.

"""

ex_poles = [r for r in records if 'ex' in r['record_type'].lower()]

pr_poles = [r for r in records if 'pr' in r['record_type'].lower() or 'pol' in r['record_type'].lower()]

matches = []

for ex in ex_poles:

for pr in pr_poles:

dist = calculate_distance(ex, pr)

if dist < max_distance:

matches.append({

'ex_pole': ex['point_id'],

'pr_pole': pr['point_id'],

'distance': dist,

'status': 'suggested' if dist < 50 else 'uncertain',

'reviewed': False

})

return matches

Route Sequencing:

python

# route_sequencer.py

def sequence_route(poles):

"""

Auto-sequence poles into design chain.

Algorithm: nearest-neighbor with bearing constraints.

"""

if not poles:

return []

# Start at lowest point_id or explicit start marker

chain = [poles[0]]

remaining = poles[1:]

while remaining:

current = chain[-1]

# Find nearest pole within bearing tolerance

candidates = []

for pole in remaining:

dist = calculate_distance(current, pole)

bearing = calculate_bearing(current, pole)

# Check bearing consistency (if not first span)

if len(chain) > 1:

prev_bearing = calculate_bearing(chain[-2], current)

bearing_diff = abs(bearing - prev_bearing)

if bearing_diff > 180:

bearing_diff = 360 - bearing_diff

# Prefer bearing consistency (<30° deviation)

confidence = 'HIGH' if bearing_diff < 30 else 'MEDIUM'

else:

confidence = 'HIGH'

candidates.append({

'pole': pole,

'distance': dist,

'bearing': bearing,

'confidence': confidence

})

# Sort by distance, pick nearest

candidates.sort(key=lambda x: x['distance'])

next_pole = candidates[0]

chain.append({

**next_pole['pole'],

'sequence_position': len(chain) + 1,

'distance_from_prev': next_pole['distance'],

'bearing_from_prev': next_pole['bearing'],

'confidence': next_pole['confidence']

})

remaining.remove(next_pole['pole'])

return chain

4. QA Validation Engine

10 Check Types:

python

# qa_engine.py

class QAEngine:

def __init__(self, rulepack='SPEN_11kV'):

self.rulepack = load_rulepack(rulepack)

self.checks = [

self.check_coordinate_bounds,

self.check_duplicate_coordinates,

self.check_height_validity,

self.check_span_distances,

self.check_angle_stay_evidence,

self.check_material_completeness,

self.check_ex_pr_clarity,

self.check_clearance_proximity,

self.check_feature_code_validity,

self.check_sequence_confidence

]

def run_all_checks(self, records):

"""

Run all QA checks on records.

Returns issues list.

"""

issues = []

for check in self.checks:

check_issues = check(records)

issues.extend(check_issues)

return issues

def check_angle_stay_evidence(self, records):

"""

Example check: angle poles should have stay evidence.

"""

issues = []

angle_poles = [r for r in records if 'angle' in r.get('function', '').lower()]

for pole in angle_poles:

# Find stay records within 20m

nearby_stays = self._find_nearby_stays(pole, records, radius=20)

if not nearby_stays:

issues.append({

'point_id': pole['point_id'],

'severity': 'BLOCKER',

'code': 'ANGLE_NO_STAY',

'category': 'mechanical',

'message': f"Angle pole {pole['point_id']} has no stay evidence captured",

'recommendation': "Check field notes/photos for stay configuration. Site revisit may be required."

})

return issues

DNO Rulepacks:

python

# dno_rules.py

RULEPACKS = {

'SPEN_11kV': {

'voltage': 11000,

'max_span_normal': 100,

'max_span_extended': 300,

'max_span_critical': 500,

'min_clearance_road': 5.8,

'min_clearance_ground': 5.2,

'stay_required_angle': 10,  # degrees

# ... 50+ rule parameters

},

'SSEN_11kV': {

'voltage': 11000,

'max_span_normal': 100,

'max_span_extended': 300,

# ... variations from SPEN

},

'NIE_11kV': {

'voltage': 11000,

'coordinate_system': 'Irish_Grid_TM65',

# ... NIE-specific rules

},

'ENWL_11kV': {

'voltage': 11000,

# ... ENWL-specific rules

}

}

5. Phase C Map Intelligence

Feature-Type Filtering (C1):

javascript

// map-viewer.js

const FILTER_BUTTONS = [

{ id: 'existing', label: 'Existing Poles', icon: '○', defaultOn: true },

{ id: 'proposed', label: 'Proposed Poles', icon: '□', defaultOn: true },

{ id: 'angle', label: 'Angle Poles', icon: '◇', defaultOn: true },

{ id: 'stays', label: 'Stays/Anchors', icon: '△', defaultOn: true },

{ id: 'context', label: 'Context/Crossings', icon: '◊', defaultOn: false },  // OFF by default

{ id: 'missingHeight', label: 'Missing Heights', icon: '⚠', defaultOn: false },

{ id: 'missingSpec', label: 'Missing Spec', icon: '⚠', defaultOn: false },

{ id: 'withRemarks', label: 'Has Remarks', icon: '📝', defaultOn: false },

{ id: 'angleNoStay', label: 'Angle Poles Missing Stays', icon: '⚠️', defaultOn: false },

{ id: 'spanAnomalies', label: 'Span Anomalies', icon: '⚠️', defaultOn: false },

{ id: 'criticalCrossings', label: 'Critical Crossings', icon: '🚦', defaultOn: false }

];

function applyFilters() {

const activeFilters = FILTER_BUTTONS.filter(f => f.active);

markers.forEach(marker => {

const feature = marker.feature;

let show = false;

for (const filter of activeFilters) {

if (matchesFilter(feature, filter)) {

show = true;

break;

}

}

marker.setOpacity(show ? 1 : 0);

});

}

Blank Field Framework (C1):

javascript

// map-viewer.js

function formatFieldForPopup(recordType, fieldName, value) {

if (fieldName === 'height') {

if (recordType === 'context') {

return null;  // Hide height field for context records

}

if (value === null || value === '') {

if (recordType.includes('ex')) {

return {

label: 'Height',

value: 'not captured',

status: 'review',

icon: '⚠️',

tooltip: 'Existing pole height should have been measured. Check field notes or plan evidence.'

};

} else if (recordType.includes('pr') || recordType.includes('pol')) {

return {

label: 'Height',

value: 'not yet specified',

status: 'info',

icon: 'ℹ️',

tooltip: 'Proposed pole specification is a design decision.'

};

}

} else {

return {

label: 'Height',

value: `${value}m`,

status: 'ok',

icon: '✓'

};

}

}

// ... similar logic for material, condition, etc.

}

Lifecycle Visualization (C2):

javascript

// map-viewer.js

function classifyLifecycleState(record, nearbyRecords) {

const recordType = record.record_type.toLowerCase();

const remarks = record.remarks?.toLowerCase() || '';

// Context records

if (recordType === 'context') return 'context';

// Existing poles

if (recordType.includes('ex')) {

if (remarks.includes('retain') || remarks.includes('keep')) {

return 'retained';

}

if (remarks.includes('recover') || remarks.includes('remove')) {

return 'recovered';

}

// Check for nearby proposed

const nearbyProposed = nearbyRecords.filter(r =>

r.record_type.toLowerCase().includes('pr') ||

r.record_type.toLowerCase().includes('pol')

);

if (nearbyProposed.length > 0) {

const closest = findClosest(record, nearbyProposed);

const dist = distance(record, closest);

if (dist < 5) return 'repositioned';

if (dist < 50) return 'replacement';  // or 'suggested'/'confirmed'

return 'unmatched_ex';

}

return 'existing';

}

// Proposed poles

if (recordType.includes('pr') || recordType.includes('pol')) {

const nearbyExisting = nearbyRecords.filter(r =>

r.record_type.toLowerCase().includes('ex')

);

if (nearbyExisting.length > 0) {

const closest = findClosest(record, nearbyExisting);

const dist = distance(record, closest);

if (dist < 50) return 'replacement';

return 'unmatched_pr';

}

return 'proposed';

}

return 'existing';  // default

}

function renderLifecycleLayer(features) {

features.forEach(feature => {

const state = feature.properties.lifecycle_state;

const marker = createMarker(feature, state);

// Add to map

lifecycleLayer.addLayer(marker);

});

// Add EX↔PR linking lines

const pairs = detectMatchedPairs(features);

pairs.forEach(pair => {

const line = L.polyline(

[[pair.ex.lat, pair.ex.lng], [pair.pr.lat, pair.pr.lng]],

{

color: pair.status === 'confirmed' ? '#3b82f6' : '#94a3b8',

weight: pair.status === 'confirmed' ? 2 : 1,

dashArray: pair.status === 'confirmed' ? null : '5, 5'

}

);

lifecycleLayer.addLayer(line);

});

}

Stay Evidence Detection (C3):

python

# qa_engine.py (backend)

def detect_angle_poles(records):

"""

Identify poles requiring stay evidence.

"""

angle_poles = []

for i, record in enumerate(records):

is_angle = False

deviation = None

# Check explicit function

if 'angle' in record.get('function', '').lower():

is_angle = True

# Check route deviation (bearing change)

if i > 0 and i < len(records) - 1:

prev_bearing = calculate_bearing(records[i-1], record)

next_bearing = calculate_bearing(record, records[i+1])

deviation = abs(next_bearing - prev_bearing)

if deviation > 180:

deviation = 360 - deviation

if deviation > 10:  # >10° = angle pole

is_angle = True

if is_angle:

angle_poles.append({

'point_id': record['point_id'],

'deviation': deviation,

'explicit': 'angle' in record.get('function', '').lower()

})

return angle_poles

def scan_for_stays(angle_pole, all_records, radius=20):

"""

Find stay records near angle pole.

"""

nearby_stays = []

for record in all_records:

record_type = record.get('record_type', '').lower()

if 'stay' in record_type or 'anchor' in record_type:

dist = distance(angle_pole, record)

if dist <= radius:

nearby_stays.append({

'point_id': record['point_id'],

'type': classify_stay_type(record),

'distance': dist,

'bearing': record.get('bearing')

})

return {

'angle_pole_id': angle_pole['point_id'],

'stays_found': len(nearby_stays),

'stay_details': nearby_stays,

'has_stay_evidence': len(nearby_stays) > 0

}

Span Anomaly Detection (C4):

python

# span_generator.py

def calculate_span_distance(pole1, pole2):

"""

Calculate 3D distance including elevation.

"""

# Horizontal distance

dx = pole2['easting'] - pole1['easting']

dy = pole2['northing'] - pole1['northing']

horizontal = math.sqrt(dx**2 + dy**2)

# Vertical distance

if 'elevation' in pole1 and 'elevation' in pole2:

dz = pole2['elevation'] - pole1['elevation']

return math.sqrt(horizontal**2 + dz**2)

return horizontal

def classify_span_status(distance, voltage=11000):

"""

Classify span based on distance and voltage.

"""

if distance < 10:

return {

'class': 'duplicate',

'severity': 'WARNING',

'message': '⚠️ Probable duplicate pole or GPS error',

'action': 'Review coordinates and field notes'

}

if voltage == 11000:  # 11kV

if distance > 500:

return {

'class': 'critical_gap',

'severity': 'BLOCKER',

'message': '🚨 Probable missing intermediate pole',

'action': 'Verify route and check for missing records'

}

elif distance > 300:

return {

'class': 'very_long',

'severity': 'WARNING',

'message': '⚠️ Very long span — verify no missing pole',

'action': 'Check field notes and survey evidence'

}

elif distance > 100:

return {

'class': 'extended',

'severity': 'INFO',

'message': 'ℹ️ Long span — verify conductor sag compliance',

'action': 'Check clearance calculations'

}

return {

'class': 'normal',

'severity': 'PASS',

'message': '✓ Typical span'

}

Crossing Context Enrichment (C4):

python

# context_crossing.py

CROSSING_TYPES = {

'road': {

'label': 'Road Crossing',

'priority': 'HIGH',

'message': 'Critical clearance check required',

'action': 'Measure statutory clearance to road surface'

},

'11xing': {

'label': '11kV Line Crossing',

'priority': 'HIGH',

'message': 'Voltage separation required',

'action': 'Ensure adequate vertical/horizontal clearance'

},

'33xing': {

'label': '33kV Line Crossing',

'priority': 'CRITICAL',

'message': 'Critical voltage clearance',

'action': 'Engineering review required before design'

},

'110xing': {

'label': '110kV Line Crossing',

'priority': 'CRITICAL',

'message': 'Specialist review required',

'action': 'DNO/ENWL approval needed, specialist design'

},

'bt': {

'label': 'BT/Telecoms Crossing',

'priority': 'MEDIUM',

'message': 'Proximity coordination required',

'action': 'Verify separation distance'

}

}

def enrich_crossing_context(crossing_record):

"""

Add priority and action guidance to crossing records.

"""

feature_code = crossing_record.get('feature_code', '').lower()

for key, crossing_type in CROSSING_TYPES.items():

if key in feature_code:

return {

**crossing_record,

'crossing_label': crossing_type['label'],

'priority': crossing_type['priority'],

'clearance_message': crossing_type['message'],

'action': crossing_type['action']

}

return {

**crossing_record,

'crossing_label': 'Crossing/Context Feature',

'priority': 'REVIEW',

'clearance_message': 'Review for potential constraints'

}

### Job Outputs (Per Processed Job)

Output Files:

uploads/jobs/<job_id>/

├── meta.json              # Summary, completeness, design-readiness

├── <original>.csv         # Original uploaded CSV

├── issues.csv             # QA flags (blocker/warning/info)

├── map_data.json          # GeoJSON point features

└── sequenced_route.json   # Design chain, EX/PR matches

Example meta.json:

json

{

"job_id": "J12946",

"filename": "2814_474c_raw_trimble_export.csv",

"processed_at": "2026-04-15T14:32:01Z",

"total_records": 91,

"mapped_features": 91,

"coordinate_system": "OSGB36",

"bounds": {

"easting_min": 365200,

"easting_max": 366800,

"northing_min": 643500,

"northing_max": 644500

},

"design_readiness": "PARTIALLY READY",

"readiness_reasons": [

"Height coverage: 19/91 poles (21%) - many heights missing or estimated",

"Material data: 0/91 poles (0%) - material not captured",

"Stay/clearance/electrical data incomplete or absent"

],

"circuit_summary": {

"total_poles": 67,

"existing_poles": 45,

"proposed_poles": 22,

"angle_poles": 12,

"terminal_poles": 2,

"intermediate_poles": 53,

"stays_anchors": 8,

"context_features": 24

},

"completeness": {

"height": "21%",

"material": "0%",

"condition": "0%",

"location": "12%",

"remarks": "45%"

},

"sequence_summary": {

"total_sequenced": 64,

"confidence_high": 32,

"confidence_medium": 32,

"confidence_low": 0,

"detached_items": 3,

"ex_pr_matches": 11

},

"qa_summary": {

"total_issues": 28,

"blockers": 0,

"warnings": 18,

"info": 10,

"pass": 63

}

}

### Real Validation Corpus

Jobs Processed (45+):

- 2814_474 family (multiple variants: 474c, 474_raw, etc.)

- 2814_513 family (controller dump format)

- Gordon family (NIE Networks, Irish Grid, split PR1/PR2)

- P010, P011 (operational jobs, real designer use)

- J12946 (SPEN, OSGB)

- Bellsprings/EWM285 (SPEN, real 11kV refurb)

- D1 test series (development validation)

Coordinate Systems Validated:

- OSGB36 (British National Grid) ✅

- Irish Grid TM65 (Northern Ireland) ✅

- WGS84 (GPS, converted on intake) ✅

DNO Rulepacks Validated:

- SPEN_11kV ✅

- SSEN_11kV ✅

- NIE_11kV ✅

- ENWL_11kV ✅

Issue Patterns (Consistent Across Jobs):

- REPL_PAIR (EX↔PR replacement relationships)

- SPAN_VERY_SHORT (<10m probable duplicates)

- ANGLE_NO_STAY (angle poles missing stay evidence)

- EXPOLE_HEIGHT_EST (existing pole heights estimated, not measured)

- HEIGHT_RANGE (heights outside plausible range)

These patterns prove GridFlow discovers real engineering uncertainties, not random noise.

## 4.2 What Doesn't Work Yet (Future Work)

### Stage 4: Field Capture + Evidence Management

NOT BUILT:

- Mobile/tablet capture interface

- Photo upload/management

- Offline workflow support

- Voice note capture

- Sketch/markup tools

- Multi-surveyor coordination

Current Workaround:

- Surveyors use Field Maps, export CSV, upload to GridFlow

- Photos stored separately (not linked in GridFlow)

- Field notes in notebook (not digital)

### Stage 5: Complete Data Model

NOT BUILT:

- 50-field popup model (only 12 fields currently)

- Pole class/strength (not captured)

- Detailed condition ratings (not captured)

- Equipment presence/ratings (sparse)

- Earthing configuration (not captured)

- Defect details (not captured)

- Wayleave/landowner info (not captured)

- Surveyor/date/GNSS accuracy (not fully surfaced)

Current State:

- Basic popups (identity, height, lifecycle, stay evidence, QA)

- Design-ready for simple jobs

- Not comprehensive enough for complex jobs

### Stage 6: GIS Integration + DNO Systems

NOT BUILT:

- ArcGIS connector

- QGIS export

- DNO asset register integration

- Field Maps round-trip

- PoleCAD direct import

- Work order integration

Current Workaround:

- Manual CSV export

- Designer imports to PoleCAD manually

- No automated DNO system updates

### Stage 7: Multi-User Deployment

NOT BUILT:

- User accounts/authentication

- Role-based access control (surveyor/reviewer/designer/approver)

- Team collaboration (comments, assignments)

- Approval workflow

- Audit logging

- Multi-project management

Current State:

- Single-user desktop app

- No authentication (local use only)

- No collaboration features

# PART 5: STRATEGIC DECISION POINT

## 5.1 The Fork in the Road

GridFlow is at a critical decision point.

Two paths forward, fundamentally different products:

### Path A: Phase C2/D (Professional QA + Display)

Product Identity:

"Enhanced pre-CAD QA gatekeeper with professional engineering workspace"

Timeline: 2-4 weeks

Scope:

- Add 10-15 key survey fields (12 → 25 field popup)

- Map UX refinement (marker size, symbol polish, layer panel split)

- Asset-specific popup layouts (existing vs. proposed vs. context)

- Professional display of survey data

- Stay as internal/consultancy tool

Investment: Modest (2-4 weeks focused development)

Risk: Low (builds on proven Phase C foundation)

Commercial Potential: Internal tool, consultancy leverage, niche productivity layer

Fields to Add (10-15 priority picks):

- Pole class/strength

- Material (expand current)

- Condition (good/fair/poor/unsafe)

- Voltage carried

- Conductor type

- Stay present (yes/no)

- Stay type (if yes)

- Lean (direction/severity)

- Defects (rot/split/burn)

- Equipment presence (transformer/switch)

- Photo indicators (linked, not uploaded)

- Surveyor/date

- GNSS accuracy

- Action required

- Wayleave/access notes

Result: 25-field professional popup (not overwhelming, design-focused)

### Path B: Stage 4 (Full Electrical Survey Platform)

Product Identity:

"Professional electrical survey-to-design platform — Field Maps replacement or complement"

Timeline: 6-12 months

Scope:

- Complete 50-field data model

- Mobile/tablet field capture

- Photo upload/management

- Evidence workflows

- Asset relationships (parent/child, connectivity)

- Multi-user collaboration

- GIS integration

- PoleCAD export

- DNO system connectors

- Commercial deployment infrastructure

Investment: Substantial (6-12 months sustained development)

Risk: High (4-5x scope expansion, unproven market demand)

Commercial Potential: Standalone commercial product, DNO procurement target, SaaS revenue

CRITICAL REQUIREMENT: Customer validation BEFORE build

- Talk to 5-10 potential customers (DNOs, contractors)

- Validate willingness to pay

- Understand must-have vs. nice-to-have features

- Secure 3-5 beta customer commitments

- Create business case (revenue model, pricing, costs)

DO NOT build Stage 4 without customer proof.

## 5.2 Decision Framework

Question 1: What is your primary goal for GridFlow?

A) Consultancy leverage / internal productivity tool → Choose Phase C2/D

- 2-4 weeks

- Low risk

- Proven foundation

- Immediate value

B) Commercial product / business venture → Consider Stage 4 → BUT ONLY AFTER customer validation

- 6-12 months

- High risk

- Needs market proof

- Customer conversations FIRST

Question 2: How much time can you invest?

A) 2-4 weeks of focused development → Choose Phase C2/D

B) 6-12 months of sustained product development → Consider Stage 4 → But validate demand first

Question 3: What customer evidence do you have?

A) None / hypothetical interest / personal need → Choose Phase C2/D first → Use in real consultancy work → Gather customer feedback → Then decide Stage 4 based on evidence

B) Real customer conversations / DNO interest / contractor demand → Consider Stage 4 → But create business case first → Revenue model, pricing, deployment cost

Question 4: What is your risk tolerance?

A) Low — prefer incremental, proven improvements → Choose Phase C2/D

B) High — willing to bet on bigger commercial opportunity → Consider Stage 4 → But validate first

## 5.3 Recommended Path: Hybrid Approach

My strong recommendation:

### Phase 1: Do Phase C2/D First (2-4 weeks)

Why:

- Builds on proven Phase C foundation

- Low risk (2-4 week timeline)

- Immediate validation on existing job corpus

- Adds 10-15 fields (professional without overwhelming)

- Proves professional display model works

- Use GridFlow in real consultancy work for 2-3 months

Deliverables:

- 25-field professional popup

- Map UX improvements (markers, symbols, layers)

- Asset-specific layouts

- Real job validation (P011, Gordon, Bellsprings +new jobs)

### Phase 2: Customer Validation (Months 2-3)

While using Phase C2/D in real work:

Talk to 5-10 potential customers:

- DNOs (SPEN, SSEN, NIE, ENWL, UKPN, NPG, etc.)

- Contractors (major electrical contractors)

- Consultancies (survey/design firms)

Ask:

- "Would you pay for this platform? How much?"

- "What features are must-have vs. nice-to-have?"

- "Would you prefer SaaS or on-premise deployment?"

- "Would you beta test?"

- "What's missing from current tools (Field Maps, PoleCAD, etc.)?"

Create business case:

- Revenue model (SaaS, license, custom deployment?)

- Pricing tiers (surveyor/designer/enterprise)

- Development cost estimate (6-12 months, £50K-200K?)

- Break-even analysis

- Market size (14 DNOs, 50+ contractors, 200+ consultancies)

Validate technical requirements:

- Do customers need full 50-field model, or is 25 enough?

- Photo upload essential, or linking sufficient?

- Mobile/tablet capture required, or desktop-only acceptable?

- Offline workflow critical, or online-only fine?

- GIS integration must-have, or nice-to-have?

### Phase 3: Decision Point (Month 4)

Review customer validation evidence:

If validation shows strong demand:

- 3+ customers commit to beta testing

- Pricing validated (customers say they'd pay)

- Business case shows path to profitability

- Feature requirements confirmed

→ Build Stage 4 roadmap with customer input → Commit to 6-12 month product development → Build survey platform properly

If validation shows modest interest:

- Customers like Phase C2/D but not willing to pay for full platform

- Feature requests don't justify 6-12 month build

- Market not ready for Field Maps replacement

→ Stay with Phase C2/D professional QA model → Continue incremental improvements based on real use → Use as consultancy leverage / internal tool → Don't expand to full platform without stronger evidence

Why This Hybrid Approach is Best:

✅ Low risk — 2-4 weeks proves professional display works ✅ Real evidence — customer conversations before 6-12 month commitment ✅ Preserves optionality — can go either direction based on validation ✅ No wasted work — Phase C2/D fields are subset of Stage 4 anyway ✅ Efficient — don't build survey platform without customer proof

The biggest mistake would be: ❌ Building Stage 4 (6-12 months) without customer validation ❌ Staying narrow forever when real commercial opportunity exists ❌ Letting survey research work sit unused because of fear of scope expansion

The safe path: ✅ Prove professional display (Phase C2/D, 2-4 weeks) ✅ Validate market demand (customer conversations, 2-3 months) ✅ Then decide Stage 4 based on real evidence

# PART 6: FILE MANIFEST FOR NEW  DESKTOP PROJECT

## 6.1 Essential Files (Tier 1 — Must Have)

These 15 files provide complete project understanding:

### Current State & Decision Context (5 files)

- GridFlow_Project-Control_Review_May_2026.md (THIS DOCUMENT)

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Single comprehensive source of truth

  - What: Complete history, current state, future vision, decision framework

  - Size: ~80,000 words

  - READ FIRST

- CRITICAL_ANALYSIS_PHASE_C_VS_SURVEY_PLATFORM_2026-05-01.md

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Defines scope decision point

  - What: Phase C2/D vs. Stage 4 analysis, 4-5x gap quantification

  - Size: ~8,600 words

- CURRENT_APP_VS_REQUIRED_STATE_ANALYSIS.md

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Current vs. required data model comparison

  - What: 12 fields → 50 fields gap analysis

  - Size: ~4,200 words

- WORKFLOW_SYSTEM.md

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Operating model across all tools

  - What: ChatGPT orchestrates, Cursor executes,  reviews

  - Size: ~2,500 words

- CHANGELOG.md

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Actual history of what shipped

  - What: Phase milestones, test count progression, commit record

### Project Identity (3 files)

- AI_CONTROL/00_PROJECT_CANONICAL.md

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: Official project identity

  - What: Product definition, repo structure, tech stack

  - Status: Outdated — needs update post-decision

- AI_CONTROL/01_CURRENT_STATE.md

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: What works, what's weak

  - What: Phase completion status, capabilities, weaknesses

  - Status: Outdated — still says "Phase 2A complete"

- AI_CONTROL/02_CURRENT_TASK.md

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: What should be worked on next

  - What: Current development focus

  - Status: Severely outdated — references Phase 1

### Phase C Evidence (2 files)

- CURSOR_BRIEF_PHASE_C.md

  - Path: /Users/noelcollins/Unitas-GridFlow/

  - Why: Complete Phase C specification (C1-C4)

  - What: Feature filtering, lifecycle viz, stay detection, span analysis

  - Status: COMPLETE — 300+ tests passing

  - Size: ~12,000 words

- AI_CONTROL/03_WORKING_RULES.md

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: AI tool operating constraints

  - What: Token efficiency, narrow tasks, cost control

### Future Specifications (2 files — ONE OR BOTH depending on decision)

- GridFlow_Phase_C_Map_Review_Detailed_Report_2026-05-01.docx (if Phase C2/D)

  - Path: /Users/noelcollins/Unitas-GridFlow/ (or validation evidence folder)

  - Why: Phase C2/D UX improvement requirements

  - What: Map polish, symbol refinement, popup improvements

  - Size: 3,924 words

  - Format: DOCX — extract text for

- UK_Electrical_Grid_Survey_Data_Capture_Report.docx (if Stage 4)

  - Path: /Users/noelcollins/Unitas-GridFlow/ (or validation evidence folder)

  - Why: Complete survey platform specification

  - What: 50-field data model, evidence management, GIS parity

  - Size: 4,544 words

  - Format: DOCX — extract text for

### Domain Context (3 files)

- AI_CONTROL/28_DOMAIN_REFERENCE_SUMMARY.md (if exists)

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: Electrical engineering domain knowledge

  - What: Feature codes, evidence quality, practitioner terminology

- AI_CONTROL/29_PRACTITIONER_REVIEW_SUMMARY.md (if exists)

  - Path: /Users/noelcollins/Unitas-GridFlow/AI_CONTROL/

  - Why: Professional language requirements

  - What: Designer-facing terminology, review standards

- P011_OPERATIONAL_FEEDBACK_2026-04-30.md (if exists)

  - Path: /Users/noelcollins/Unitas-GridFlow/ (or validation evidence folder)

  - Why: Real designer use, operational blockers

  - What: 6 blockers identified, all resolved in Phase C

## 6.2 Optional Reference Files (Tier 2 — As Needed)

### Technical Implementation (read if discussing code)

- app/electrical_schema.py

  - Asset taxonomy (6 families, equipment types)

  - ~420 lines

- app/survey_connectivity.py

  - EX↔PR matching, connectivity normalization

  - ~380 lines

- app/qa_engine.py

  - Validation rules, issue generation

  - ~1,095 lines

- app/route_sequencer.py

  - Auto-sequencing, confidence levels

  - ~691 lines

- app/static/js/map-viewer.js

  - Map intelligence, filters, popups

  - ~2,809 lines

  - SENSITIVE — treat as fragile

### Validation Evidence (read if discussing specific jobs)

- uploads/projects/P005/files/F001/meta.json

  - P005/F001 job summary (157 records, partially ready)

- uploads/jobs/J12946/meta.json

  - J12946 job summary (91 records, 2814_474c family)

- validation_data/gordon_pt1/ (selective files)

  - Gordon raw survey + PR1/PR2 splits

  - NIE Networks, Irish Grid validation

- validation_data/2814_513/ (selective files)

  - Raw controller dump format

  - Completeness reporting focus

### DNO Rules & Regional Context

- app/dno_rules.py

  - DNO rulepack definitions (SPEN, SSEN, NIE, ENWL)

  - Voltage-specific parameters

  - ~200 lines

## 6.3 DO NOT Include in Project

These will cause confusion or waste tokens:

❌ _archive/ directory — historical only, outdated ❌ Full uploads/ tree — too large, unnecessary ❌ All validation evidence — selective examples only ❌ .venv312/ or .env — local environment, not project truth ❌ All test files — reference only when needed ❌ Old strategic reviews — pre-Phase C, superseded ❌ Previous control review (undated) — replaced by May 2026 edition

## 6.4 Custom Instructions for  Desktop Project

Add these as project knowledge or custom instructions:

PROJECT CONTEXT:

You are the orchestrator for Unitas-GridFlow, a survey-to-design intelligence

platform for UK electrical overhead line (OHL) work.

CURRENT STATUS (May 2026):

- Phase C complete (C1-C4): 300+ tests passing

- Feature filtering, lifecycle viz, stay detection, span analysis operational

- Real validation: P011, Gordon, Bellsprings (DNO jobs)

- Strategic decision pending: Phase C2/D vs. Stage 4

CRITICAL DECISION POINT:

GridFlow is choosing between two fundamentally different products:

Path A (Phase C2/D): Professional QA + Display Platform

- Timeline: 2-4 weeks

- Scope: Add 10-15 key fields (12 → 25 field popup), map UX refinement

- Identity: Enhanced pre-CAD QA gatekeeper

- Risk: LOW, builds on proven foundation

Path B (Stage 4): Full Electrical Survey Platform

- Timeline: 6-12 months

- Scope: 50-field data model, mobile capture, evidence management, GIS integration

- Identity: Professional survey platform, Field Maps replacement potential

- Risk: HIGH, needs customer validation FIRST

DECISION PENDING: No implementation work until strategic direction confirmed.

YOUR ROLE:

- Strategic planning and decision analysis

- Specification review and domain alignment

- Control layer maintenance

- Customer validation guidance

YOU DO NOT:

- Write implementation code (Cursor does that)

- Execute in terminal ( Code does that)

- Make final strategic decisions (Noel does that)

KEY PRINCIPLES:

- Validation-led development (real operational use only)

- Scope discipline (Phase C2/D ≠ Stage 4)

- Token efficiency (read minimum necessary files)

- Real evidence gates all decisions

DOMAIN CONTEXT:

- UK electrical distribution (11kV, 33kV, 132kV OHL)

- 4 DNOs validated: NIE, SPEN, SSEN, ENWL

- 3 coordinate systems: OSGB36, Irish Grid TM65, WGS84

- 6 asset families: poles, spans, stays, cables, context, equipment

- 11 lifecycle states: existing, proposed, retained, recovered, replacement, etc.

CURRENT DATA MODEL:

- Existing: ~12 fields across 8 popup sections

- Required (professional): 25-35 fields (Phase C2/D)

- Required (full platform): 50+ fields (Stage 4)

- Gap: 2-4x expansion depending on path chosen

VALIDATION CORPUS:

- P011 (operational feedback, 6 blockers resolved)

- Gordon (NIE, Irish Grid, split-file testing)

- Bellsprings/EWM285 (SPEN, real 11kV refurb)

- 2814_474/513 families (regression baselines)

- 45+ processed jobs total

CONTROL LAYER PRIORITY:

1. GridFlow_Project-Control_Review_May_2026.md (THIS IS TRUTH)

2. CRITICAL_ANALYSIS documents (decision framework)

3. CURSOR_BRIEF_PHASE_C.md (Phase C complete)

4. AI_CONTROL files (need update post-decision)

5. CHANGELOG.md (actual shipped history)

FILE SENSITIVITY:

- map-viewer.js (~2,809 lines): FRAGILE, narrow changes only

- qa_engine.py (~1,095 lines): stable, well-tested

- Control files: update after major milestones only

RECOMMENDED NEXT STEP:

1. Read GridFlow_Project-Control_Review_May_2026.md (complete context)

2. Read CRITICAL_ANALYSIS (decision framework)

3. Help Noel decide: Phase C2/D or Stage 4?

4. If Phase C2/D: create specification from Map Review doc

5. If Stage 4: guide customer validation process BEFORE build

# PART 7: SUMMARY & NEXT ACTIONS

## 7.1 What You Now Have

This document provides:

✅ Complete project history — from inception → Phase C complete ✅ Technical implementation details — stack, architecture, data model, code structure ✅ Domain knowledge — UK electrical OHL, DNOs, asset types, survey workflows ✅ Current state — what works (Phase C), what doesn't (Stage 4+) ✅ Future vision — what GridFlow will be when fully developed ✅ Strategic decision framework — Phase C2/D vs. Stage 4, customer validation ✅ File manifest — 15 essential files + optional references for new  Project ✅ Custom instructions — project context, principles, domain, control layer

Everything a new  Desktop Project needs to understand GridFlow completely.

## 7.2 Immediate Next Actions

### For You (Noel)

1. Strategic Decision (This Week)

Choose path:

- Phase C2/D (2-4 weeks, professional QA+display)

- Stage 4 (6-12 months, full platform, validate first)

- Hybrid (Phase C2/D → customer validation → then decide Stage 4)

Record decision:

- Create AI_CONTROL/07_STRATEGIC_DECISION_MAY_2026.md

- Document rationale, chosen path, timeline

2. Control Layer Refresh (Next Week)

Update these files based on decision:

- AI_CONTROL/00_PROJECT_CANONICAL.md (product identity)

- AI_CONTROL/01_CURRENT_STATE.md (Phase C complete status)

- AI_CONTROL/02_CURRENT_TASK.md (chosen path, next spec)

- README.md (product description)

- CHANGELOG.md (Phase C completion entry)

3. New  Desktop Project Setup (When Ready)

Create project folder:

- Include 15 essential files (Tier 1)

- Add custom instructions (project context)

- Test: can  answer the 10 key questions?

Validate understanding:

- "What is GridFlow?"

- "What phase is it in?"

- "What's the current decision point?"

- "What are the two path options?"

- "What validation is required for Stage 4?"

### For  Desktop (Me)

If Phase C2/D Chosen:

1. Review Map Review Document

- Extract UX improvement requirements

- Identify 10-15 priority fields to add

- Create asset-specific popup layouts

2. Create Phase C2/D Specification

- Map UX refinements (marker size, symbols, layers)

- Field additions (12 → 25 fields)

- Popup organization (existing/proposed/context layouts)

- Test requirements

- Visual acceptance criteria

3. Support Implementation

- Review Cursor output for domain alignment

- Check terminology (practitioner-facing language)

- Validate against operational evidence (P011, Gordon)

If Stage 4 Chosen:

1. Guide Customer Validation Process

- Create customer interview template

- Identify target customers (DNOs, contractors, consultancies)

- Develop business case framework

- Define feature priority questions

2. Review Survey Research Document

- Extract 50-field data model requirements

- Identify must-have vs. nice-to-have features

- Create Stage 4 roadmap outline

3. Support Commercial Analysis

- Review market size estimates

- Validate pricing model assumptions

- Assess competitive landscape

BUT: Do NOT create Stage 4 specification until customer validation complete.

## 7.3 Success Criteria

New  Desktop Project is successful if it can:

✅ Understand project history — MVP → Phase C evolution ✅ Explain current state — 300+ tests, Phase C complete, decision pending ✅ Describe future vision — full survey platform (Stage 4+) ✅ Articulate decision point — Phase C2/D vs. Stage 4, validation requirements ✅ Provide domain context — UK OHL, DNOs, asset types, workflows ✅ Guide strategic decisions — customer validation, business case, risk assessment ✅ Review specifications — domain alignment, practitioner terminology ✅ Maintain control layer — update canonical files after milestones

It should NOT: ❌ Write implementation code (Cursor's job) ❌ Execute terminal commands ( Code's job) ❌ Make final strategic decisions (Noel's authority) ❌ Recommend Stage 4 without customer validation ❌ Conflate Phase C2/D scope with Stage 4 scope

## 7.4 Final Note

GridFlow is a real, valuable product with a clear path forward.

Phase C proved the technical foundation is solid:

- 300+ tests passing

- Real operational validation

- Professional map intelligence

- Design-ready outputs

The next phase is not a technical question — it's a strategic decision:

Phase C2/D: Low-risk professional enhancement (2-4 weeks) Stage 4: High-reward commercial platform (6-12 months, validate first)

Both are valid. Both can succeed. The choice depends on your goals, timeline, and customer evidence.

This document gives you — and any new  Desktop Project — everything needed to make that decision with full context.

END OF MASTER PROJECT BRIEFING